Boom lift checklist

Date __________________________________   Time __________________________________

Site or location __________________________________   Operator name __________________________________

Boom type, make and unit number __________________________________   Hour meter reading __________________________________

Walk around the machine

| Item | Pass | Fail | N/A |
| --- | --- | --- | --- |
| Manual, load chart and rating plate are on the machine and readable | ☐ | ☐ | ☐ |
| Basket floor, guardrails, gate and toe boards are secure and undamaged | ☐ | ☐ | ☐ |
| Boom sections, pins and wear pads show no cracks, bends or missing parts | ☐ | ☐ | ☐ |
| Hydraulic hoses, cylinders and fittings are free of leaks | ☐ | ☐ | ☐ |
| Turntable and swing gear show no damage, and the swing area is clear | ☐ | ☐ | ☐ |
| Tires, wheels or tracks are in good condition | ☐ | ☐ | ☐ |
| Axles, outriggers or stabilizers extend, retract and lock, where fitted | ☐ | ☐ | ☐ |
| Fall protection anchor points in the basket are intact | ☐ | ☐ | ☐ |
| Battery, fuel and fluid levels are correct | ☐ | ☐ | ☐ |
| Decals, warnings and control labels are in place and readable | ☐ | ☐ | ☐ |

Test the controls

| Item | Pass | Fail | N/A |
| --- | --- | --- | --- |
| Ground controls work and override the basket controls | ☐ | ☐ | ☐ |
| Basket controls work, and each moves the boom the way its label says | ☐ | ☐ | ☐ |
| Emergency stop works at both stations | ☐ | ☐ | ☐ |
| Emergency lowering works | ☐ | ☐ | ☐ |
| Horn, travel alarm and lights work | ☐ | ☐ | ☐ |
| Tilt alarm and any limit or interlock devices work | ☐ | ☐ | ☐ |
| Drive and steering work, and the brakes hold | ☐ | ☐ | ☐ |

Check the work area

| Item | Pass | Fail | N/A |
| --- | --- | --- | --- |
| Ground is firm and level, with no holes, slopes, soft spots or drop-offs | ☐ | ☐ | ☐ |
| Overhead hazards and power lines are identified, and the required distance is kept | ☐ | ☐ | ☐ |
| Area under the boom and swing path is barricaded, or a ground person is in place | ☐ | ☐ | ☐ |
| Wind and weather are within the machine's limits | ☐ | ☐ | ☐ |
| Operator is trained on this type of lift and wears the fall protection the maker calls for | ☐ | ☐ | ☐ |
| A rescue plan is in place for the person in the basket | ☐ | ☐ | ☐ |

Defects found and action taken

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

Taken out of service?  ☐ Yes   ☐ No        Reported to ______________________

Inspector signature __________________________________   Date __________________________________

Supervisor review __________________________________   Date __________________________________

A starting point. Adapt it to your equipment, your procedures and the rules that apply to you. Free template from portalbaseai.com