Crane and rigging checklist

Date __________________________________   Time __________________________________

Site or location __________________________________   Operator name __________________________________

Crane type and unit number __________________________________   Rigger or signal person __________________________________

Rigging gear

| Item | Pass | Fail | N/A |
| --- | --- | --- | --- |
| Slings show no cuts, broken strands, kinks or heat damage | ☐ | ☐ | ☐ |
| Sling tags are attached and readable | ☐ | ☐ | ☐ |
| Shackles have the right pin, seated and secured | ☐ | ☐ | ☐ |
| Hooks are not bent, twisted or opened up | ☐ | ☐ | ☐ |
| Hook latches are present and close | ☐ | ☐ | ☐ |
| Rigging is rated for the load, per the tag and the current standard | ☐ | ☐ | ☐ |

Crane

| Item | Pass | Fail | N/A |
| --- | --- | --- | --- |
| Hoist and wire rope show no kinks, crushing or broken wires, per the maker's manual | ☐ | ☐ | ☐ |
| Wire rope is seated on the drum and sheaves | ☐ | ☐ | ☐ |
| Hydraulic hoses and cylinders are free of leaks | ☐ | ☐ | ☐ |
| Controls, brakes and limit devices work | ☐ | ☐ | ☐ |
| Horn, alarms and lights work | ☐ | ☐ | ☐ |
| Load chart is in the cab and readable | ☐ | ☐ | ☐ |
| Outriggers extend fully with pads under them, if used | ☐ | ☐ | ☐ |

Ground and lift plan

| Item | Pass | Fail | N/A |
| --- | --- | --- | --- |
| Ground is firm and level under the crane and the pads | ☐ | ☐ | ☐ |
| Weight of the load is known, and it's within the load chart for this setup | ☐ | ☐ | ☐ |
| Power lines and overhead hazards are identified and kept clear | ☐ | ☐ | ☐ |
| Wind and weather are within the maker's limits | ☐ | ☐ | ☐ |

Signals and area

| Item | Pass | Fail | N/A |
| --- | --- | --- | --- |
| One signal person is named, and the operator knows who | ☐ | ☐ | ☐ |
| Signals or radio channel are agreed before the lift | ☐ | ☐ | ☐ |
| Swing path and drop zone are barricaded | ☐ | ☐ | ☐ |
| Nobody stands under a load | ☐ | ☐ | ☐ |

Defects found and action taken

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

Taken out of service?  ☐ Yes   ☐ No        Reported to ______________________

Inspector signature __________________________________   Date __________________________________

Supervisor review __________________________________   Date __________________________________

A starting point. Adapt it to your equipment, your procedures and the rules that apply to you. Free template from portalbaseai.com